# Migrating Open5GS UPF to Kubernetes

Containerizing 3 Slice-Specific UPFs (eMBB, URLLC, mMTC)
in a Minikube Cluster on a Single VM

Date: February 22, 2026
Environment: Ubuntu 22.04 / Open5GS v2.7.6 / Minikube v1.35.1

# Table of Contents

1. Overview

2. Environment & Starting State

3. Pre-Implementation Fixes

3.1 iptables Cleanup

3.2 Secondary IP Addresses

3.3 MongoDB Repair

4. Docker Image

4.1 Dockerfile

4.2 Entrypoint Script

4.3 Build Command

5. Kubernetes Manifests

5.1 eMBB UPF

5.2 URLLC UPF

5.3 mMTC UPF

6. Deployment Commands

7. Issues Encountered & Fixes

7.1 GTP-U Bind Failure

7.2 TUN Device Conflict

7.3 mMTC ResourceQuota

8. Verification

9. Rollback Procedure

10. Architecture Diagram

# 1. Overview

This document describes the process of migrating three slice-specific Open5GS UPF (User Plane Function) instances from native systemd services to Kubernetes pods. The UPFs serve three network slices:

| Slice | SST | DNN | Subnet | tc Rate |
| --- | --- | --- | --- | --- |
| eMBB | 1 | internet | 10.45.0.0/24 | 100mbit |
| URLLC | 2 | urllc | 10.46.0.0/24 | 50mbit + 1ms |
| mMTC | 3 | iot | 10.47.0.0/24 | 1mbit (ceil 5) |

The control plane (AMF, NRF, NSSF, SMFs, etc.) remains running natively on the host. Only the UPFs are moved into Kubernetes pods using hostNetwork mode, so the SMF configurations require no changes — they continue to communicate via the same PFCP addresses on the loopback interface.

# 2. Environment & Starting State

| OS | Ubuntu 22.04 (kernel 6.8.0-90) |
| --- | --- |
| Open5GS | v2.7.6 |
| Minikube | v1.35.1 (docker driver) |
| Kubernetes | v1.35.1 (single-node) |
| VM IP | 192.168.49.143 (ens33) |
| Minikube IP | 192.168.58.2 (bridge br-7aeac7398ce9) |
| Docker | v28.2.2 |
| Namespaces | embb, urllc, mmtc (pre-existing) |

## Native UPF Configuration (Before Migration)

| UPF | PFCP Address | GTP-U Address | TUN Device | Systemd Unit |
| --- | --- | --- | --- | --- |
| eMBB | 127.0.0.7:8805 | 192.168.49.143:2152 | ogstun-embb | open5gs-upfd-embb |
| URLLC | 127.0.0.8:8806 | 192.168.49.144:2152 | ogstun-urllc | open5gs-upfd-urllc |
| mMTC | 127.0.0.9:8807 | 192.168.49.146:2152 | ogstun-mmtc | open5gs-upfd-mmtc |

# 3. Pre-Implementation Fixes

## 3.1 iptables Cleanup

The existing iptables rules had ~180+ duplicate entries across the nat, filter, and mangle tables (accumulated from repeated script runs). These were cleaned down to 15 essential rules while preserving all functional rules.

Commands executed:

# Backup existing rules

sudo cp /etc/iptables/rules.v4 /etc/iptables/rules.v4.backup

# Flush all tables

sudo iptables -t mangle -F

sudo iptables -t mangle -X PDU_MARK

sudo iptables -t nat -F

sudo iptables -t filter -F

# Load clean rules

sudo iptables-restore < /tmp/clean-rules.v4

# Copy clean rules to persistent file

sudo cp /tmp/clean-rules.v4 /etc/iptables/rules.v4

Clean rules preserved:

| Table | Chain | Rule |
| --- | --- | --- |
| nat | POSTROUTING | MASQUERADE for ens33 (general) |
| nat | POSTROUTING | MASQUERADE for 10.45/46/47.0.0/16 subnets |
| mangle | PDU_MARK | MARK 0xa/0x14/0x1e per slice subnet |
| mangle | PREROUTING/POST | CONNMARK save/restore for tc |
| filter | FORWARD | ACCEPT (single rule) |

## 3.2 Secondary IP Addresses

The URLLC and mMTC UPFs require GTP-U endpoints on 192.168.49.144 and 192.168.49.146 respectively. These secondary IPs were not configured on ens33.

sudo ip addr add 192.168.49.144/24 dev ens33

sudo ip addr add 192.168.49.146/24 dev ens33

Note: These IPs are runtime-only and must be re-added after reboot.

## 3.3 MongoDB Repair

MongoDB was crashing with SIGABRT due to a corrupted WiredTiger lock file from an unclean shutdown. This affected HSS, PCF, PCRF, UDR, and WebUI services.

sudo systemctl stop mongod

sudo rm /var/lib/mongodb/mongod.lock

sudo mongod --dbpath /var/lib/mongodb --repair

sudo chown -R mongodb:mongodb /var/lib/mongodb

sudo systemctl start mongod

# 4. Docker Image

## 4.1 Dockerfile

Location: /etc/open5gs/k8s/Dockerfile

File: /etc/open5gs/k8s/Dockerfile

FROM ubuntu:22.04

ENV DEBIAN_FRONTEND=noninteractive

RUN apt-get update && apt-get install -y \

software-properties-common \

gnupg \

curl \

&& add-apt-repository ppa:open5gs/latest \

&& apt-get update \

&& apt-get install -y \

open5gs-upf=2.7.6~jammy \

iproute2 \

iptables \

iputils-ping \

net-tools \

&& rm -rf /var/lib/apt/lists/*

COPY entrypoint.sh /entrypoint.sh

RUN chmod +x /entrypoint.sh

ENTRYPOINT ["/entrypoint.sh"]

## 4.2 Entrypoint Script

Location: /etc/open5gs/k8s/entrypoint.sh

This script runs inside the container before the UPF starts. It creates the TUN device, enables IP forwarding, sets up NAT, and applies tc traffic shaping.

File: /etc/open5gs/k8s/entrypoint.sh

#!/bin/bash

set -e

# --- Environment variables (set from K8s manifests) ---

TUN_DEV=${TUN_DEV:-ogstun}

UE_SUBNET=${UE_SUBNET:-10.45.0.0/24}

UE_GATEWAY=${UE_GATEWAY:-10.45.0.1/24}

UPF_CONFIG=${UPF_CONFIG:-/etc/open5gs/upf.yaml}

# Slice-specific tc shaping (env vars from deployment)

TC_RATE=${TC_RATE:-}

TC_CEIL=${TC_CEIL:-}

TC_LATENCY=${TC_LATENCY:-}

# --- 1. Create TUN device ---

echo "[entrypoint] Creating TUN device: $TUN_DEV"

if ! ip link show "$TUN_DEV" &>/dev/null; then

ip tuntap add name "$TUN_DEV" mode tun

fi

ip addr add "$UE_GATEWAY" dev "$TUN_DEV" 2>/dev/null || true

ip link set "$TUN_DEV" up

# --- 2. Enable IP forwarding + NAT ---

echo "[entrypoint] Setting up IP forwarding and NAT"

sysctl -w net.ipv4.ip_forward=1

iptables -t nat -C POSTROUTING -s "$UE_SUBNET" ! -o "$TUN_DEV" -j MASQUERADE 2>/dev/null \

|| iptables -t nat -A POSTROUTING -s "$UE_SUBNET" ! -o "$TUN_DEV" -j MASQUERADE

# --- 3. Apply tc shaping (if configured) ---

if [ -n "$TC_RATE" ]; then

echo "[entrypoint] Applying tc shaping: rate=$TC_RATE ceil=${TC_CEIL:-$TC_RATE}"

tc qdisc del dev "$TUN_DEV" root 2>/dev/null || true

tc qdisc add dev "$TUN_DEV" root handle 1: htb default 1

tc class add dev "$TUN_DEV" parent 1: classid 1:1 htb \

rate "$TC_RATE" ceil "${TC_CEIL:-$TC_RATE}"

if [ -n "$TC_LATENCY" ]; then

echo "[entrypoint] Adding netem: latency=$TC_LATENCY"

tc qdisc add dev "$TUN_DEV" parent 1:1 handle 10: netem \

delay "$TC_LATENCY"

fi

fi

# --- 4. Launch UPF ---

echo "[entrypoint] Starting open5gs-upfd with config: $UPF_CONFIG"

echo "[entrypoint] TUN=$TUN_DEV SUBNET=$UE_SUBNET GATEWAY=$UE_GATEWAY"

exec open5gs-upfd -c "$UPF_CONFIG"

## 4.3 Build Command

# Point docker CLI to minikube's docker daemon

eval $(minikube docker-env)

# Build the image (inside minikube so pods can pull it locally)

docker build -t open5gs-upf:local /etc/open5gs/k8s/

# Verify

docker images | grep open5gs-upf

# 5. Kubernetes Manifests

Each slice has a single YAML file containing three resources: a ConfigMap (UPF configuration), a Deployment (privileged pod with hostNetwork), and a ClusterIP Service (for metrics).

## 5.1 eMBB UPF Manifest

Location: /etc/open5gs/k8s/embb/upf-embb.yaml

File: /etc/open5gs/k8s/embb/upf-embb.yaml

---

# ============================================================

# eMBB UPF — ConfigMap

# ============================================================

apiVersion: v1

kind: ConfigMap

metadata:

name: upf-embb-config

namespace: embb

data:

upf-embb.yaml: |

logger:

file:

path: /var/log/open5gs/upf-embb.log

level: info

global:

max:

ue: 1024

upf:

pfcp:

server:

- address: 127.0.0.7

port: 8805

gtpu:

server:

- address: 192.168.49.143

port: 2152

session:

- subnet: 10.45.0.0/24

gateway: 10.45.0.1

dev: ogstun-embb

dnn: internet

---

# ============================================================

# eMBB UPF — Deployment

# ============================================================

apiVersion: apps/v1

kind: Deployment

metadata:

name: upf-embb

namespace: embb

labels:

app: upf-embb

slice: embb

spec:

replicas: 1

selector:

matchLabels:

app: upf-embb

template:

metadata:

labels:

app: upf-embb

slice: embb

spec:

hostNetwork: true

dnsPolicy: ClusterFirstWithHostNet

containers:

- name: upf

image: open5gs-upf:local

imagePullPolicy: Never

securityContext:

privileged: true

capabilities:

add: ["NET_ADMIN", "SYS_ADMIN"]

env:

- name: TUN_DEV

value: "ogstun-embb"

- name: UE_SUBNET

value: "10.45.0.0/24"

- name: UE_GATEWAY

value: "10.45.0.1/24"

- name: UPF_CONFIG

value: "/config/upf-embb.yaml"

# tc shaping: eMBB = high bandwidth

- name: TC_RATE

value: "100mbit"

- name: TC_CEIL

value: "100mbit"

volumeMounts:

- name: config

mountPath: /config

- name: dev-net-tun

mountPath: /dev/net/tun

resources:

requests:

memory: "128Mi"

cpu: "100m"

limits:

memory: "512Mi"

cpu: "500m"

volumes:

- name: config

configMap:

name: upf-embb-config

- name: dev-net-tun

hostPath:

path: /dev/net/tun

type: CharDevice

---

# ============================================================

# eMBB UPF — Metrics Service

# ============================================================

apiVersion: v1

kind: Service

metadata:

name: upf-embb-metrics

namespace: embb

labels:

app: upf-embb

slice: embb

spec:

type: ClusterIP

selector:

app: upf-embb

ports:

- name: metrics

port: 9090

targetPort: 9090

## 5.2 URLLC UPF Manifest

Location: /etc/open5gs/k8s/urllc/upf-urllc.yaml

File: /etc/open5gs/k8s/urllc/upf-urllc.yaml

---

# ============================================================

# URLLC UPF — ConfigMap

# ============================================================

apiVersion: v1

kind: ConfigMap

metadata:

name: upf-urllc-config

namespace: urllc

data:

upf-urllc.yaml: |

logger:

file:

path: /var/log/open5gs/upf-urllc.log

level: info

global:

max:

ue: 1024

upf:

pfcp:

server:

- address: 127.0.0.8

port: 8806

gtpu:

server:

- address: 192.168.49.144

port: 2152

session:

- subnet: 10.46.0.0/24

gateway: 10.46.0.1

dev: ogstun-urllc

dnn: urllc

---

# ============================================================

# URLLC UPF — Deployment

# ============================================================

apiVersion: apps/v1

kind: Deployment

metadata:

name: upf-urllc

namespace: urllc

labels:

app: upf-urllc

slice: urllc

spec:

replicas: 1

selector:

matchLabels:

app: upf-urllc

template:

metadata:

labels:

app: upf-urllc

slice: urllc

spec:

hostNetwork: true

dnsPolicy: ClusterFirstWithHostNet

containers:

- name: upf

image: open5gs-upf:local

imagePullPolicy: Never

securityContext:

privileged: true

capabilities:

add: ["NET_ADMIN", "SYS_ADMIN"]

env:

- name: TUN_DEV

value: "ogstun-urllc"

- name: UE_SUBNET

value: "10.46.0.0/24"

- name: UE_GATEWAY

value: "10.46.0.1/24"

- name: UPF_CONFIG

value: "/config/upf-urllc.yaml"

# tc shaping: URLLC = moderate bandwidth + ultra-low latency

- name: TC_RATE

value: "50mbit"

- name: TC_CEIL

value: "50mbit"

- name: TC_LATENCY

value: "1ms"

volumeMounts:

- name: config

mountPath: /config

- name: dev-net-tun

mountPath: /dev/net/tun

resources:

requests:

memory: "128Mi"

cpu: "100m"

limits:

memory: "512Mi"

cpu: "500m"

volumes:

- name: config

configMap:

name: upf-urllc-config

- name: dev-net-tun

hostPath:

path: /dev/net/tun

type: CharDevice

---

# ============================================================

# URLLC UPF — Metrics Service

# ============================================================

apiVersion: v1

kind: Service

metadata:

name: upf-urllc-metrics

namespace: urllc

labels:

app: upf-urllc

slice: urllc

spec:

type: ClusterIP

selector:

app: upf-urllc

ports:

- name: metrics

port: 9091

targetPort: 9091

## 5.3 mMTC UPF Manifest

Location: /etc/open5gs/k8s/mmtc/upf-mmtc.yaml

File: /etc/open5gs/k8s/mmtc/upf-mmtc.yaml

---

# ============================================================

# mMTC UPF — ConfigMap

# ============================================================

apiVersion: v1

kind: ConfigMap

metadata:

name: upf-mmtc-config

namespace: mmtc

data:

upf-mmtc.yaml: |

logger:

file:

path: /var/log/open5gs/upf-mmtc.log

level: info

global:

max:

ue: 1024

upf:

pfcp:

server:

- address: 127.0.0.9

port: 8807

gtpu:

server:

- address: 192.168.49.146

port: 2152

session:

- subnet: 10.47.0.0/24

gateway: 10.47.0.1

dev: ogstun-mmtc

dnn: iot

---

# ============================================================

# mMTC UPF — Deployment

# ============================================================

apiVersion: apps/v1

kind: Deployment

metadata:

name: upf-mmtc

namespace: mmtc

labels:

app: upf-mmtc

slice: mmtc

spec:

replicas: 1

selector:

matchLabels:

app: upf-mmtc

template:

metadata:

labels:

app: upf-mmtc

slice: mmtc

spec:

hostNetwork: true

dnsPolicy: ClusterFirstWithHostNet

containers:

- name: upf

image: open5gs-upf:local

imagePullPolicy: Never

securityContext:

privileged: true

capabilities:

add: ["NET_ADMIN", "SYS_ADMIN"]

env:

- name: TUN_DEV

value: "ogstun-mmtc"

- name: UE_SUBNET

value: "10.47.0.0/24"

- name: UE_GATEWAY

value: "10.47.0.1/24"

- name: UPF_CONFIG

value: "/config/upf-mmtc.yaml"

# tc shaping: mMTC = low bandwidth for IoT

- name: TC_RATE

value: "1mbit"

- name: TC_CEIL

value: "5mbit"

volumeMounts:

- name: config

mountPath: /config

- name: dev-net-tun

mountPath: /dev/net/tun

resources:

requests:

memory: "64Mi"

cpu: "50m"

limits:

memory: "256Mi"

cpu: "300m"

volumes:

- name: config

configMap:

name: upf-mmtc-config

- name: dev-net-tun

hostPath:

path: /dev/net/tun

type: CharDevice

---

# ============================================================

# mMTC UPF — Metrics Service

# ============================================================

apiVersion: v1

kind: Service

metadata:

name: upf-mmtc-metrics

namespace: mmtc

labels:

app: upf-mmtc

slice: mmtc

spec:

type: ClusterIP

selector:

app: upf-mmtc

ports:

- name: metrics

port: 9092

targetPort: 9092

# 6. Deployment Commands

## Step 1: Build Docker Image

eval $(minikube docker-env)

docker build -t open5gs-upf:local /etc/open5gs/k8s/

## Step 2: Stop Native UPF Services

sudo systemctl stop open5gs-upfd-embb open5gs-upfd-urllc open5gs-upfd-mmtc

sudo systemctl disable open5gs-upfd-embb open5gs-upfd-urllc open5gs-upfd-mmtc

## Step 3: Add GTP-U IPs Inside Minikube Container

Because minikube uses the docker driver, the Kubernetes node is a Docker container with its own network (192.168.58.2). The GTP-U IPs must be added inside this container so UPF pods with hostNetwork can bind to them.

sudo docker exec minikube ip addr add 192.168.49.143/24 dev eth0

sudo docker exec minikube ip addr add 192.168.49.144/24 dev eth0

sudo docker exec minikube ip addr add 192.168.49.146/24 dev eth0

## Step 4: Deploy UPF Pods

kubectl apply -f /etc/open5gs/k8s/embb/upf-embb.yaml

kubectl apply -f /etc/open5gs/k8s/urllc/upf-urllc.yaml

kubectl apply -f /etc/open5gs/k8s/mmtc/upf-mmtc.yaml

## Step 5: Verify Pods Running

kubectl get pods -n embb -l app=upf-embb

kubectl get pods -n urllc -l app=upf-urllc

kubectl get pods -n mmtc -l app=upf-mmtc

# 7. Issues Encountered & Fixes

## 7.1 GTP-U Bind Failure (Cannot assign requested address)

Problem:

UPF pods crashed because they tried to bind GTP-U to 192.168.49.x addresses, but with minikube's docker driver, hostNetwork gives pods minikube's network (192.168.58.2), not the VM's ens33.

Error message:

socket bind(2) [192.168.49.143]:2152 failed (99:Cannot assign requested address)

Fix: Added the GTP-U IPs inside the minikube container:

sudo docker exec minikube ip addr add 192.168.49.143/24 dev eth0

## 7.2 TUN Device Conflict (Device or resource busy)

Problem:

All 3 UPF pods share the host network. Each UPF tried to create the default "ogstun" TUN device, but only the first pod (eMBB) could claim it.

Error message:

ioctl() failed : dev[/dev/net/tun] flags[0x1000] (16:Device or resource busy)

tun_open(dev:ogstun) failed

Fix: Added dev: field to each UPF's session config in the ConfigMap:

# eMBB ConfigMap session block:

session:

- subnet: 10.45.0.0/24

gateway: 10.45.0.1

dev: ogstun-embb      # <-- added this

dnn: internet

## 7.3 mMTC ResourceQuota Exceeded

Problem:

The mmtc namespace had a ResourceQuota (limits.cpu=500m, limits.memory=512Mi). The existing mmtc-app pod used 100m/128Mi, and the UPF requested 500m/512Mi, exceeding the total.

Fix: Reduced UPF resource limits to cpu: 300m, memory: 256Mi

# 8. Verification

## Pod Status

$ kubectl get pods -A -l slice

NAMESPACE   NAME                        READY   STATUS    AGE

embb        upf-embb-76859dd8db-25f2q   1/1     Running   ✅

urllc       upf-urllc-9c7f584d8-bng47   1/1     Running   ✅

mmtc        upf-mmtc-595577d54d-jdsqh    1/1     Running   ✅

## UPF Initialization Logs

# eMBB

pfcp_server() [127.0.0.7]:8805

gtp_server() [192.168.49.143]:2152

UPF initialize...done

# URLLC

pfcp_server() [127.0.0.8]:8806

gtp_server() [192.168.49.144]:2152

UPF initialize...done

# mMTC

pfcp_server() [127.0.0.9]:8807

gtp_server() [192.168.49.146]:2152

UPF initialize...done

## tc Shaping Verification

$ kubectl exec -n embb  deploy/upf-embb  -- tc qdisc show dev ogstun-embb

qdisc htb 1: root r2q 10 default 0x1

$ kubectl exec -n urllc deploy/upf-urllc -- tc qdisc show dev ogstun-urllc

qdisc htb 1: root r2q 10 default 0x1

qdisc netem 10: parent 1:1 delay 1ms

$ kubectl exec -n mmtc  deploy/upf-mmtc  -- tc qdisc show dev ogstun-mmtc

qdisc htb 1: root r2q 10 default 0x1

## Verification Commands

# Check pod logs

kubectl logs -n embb deploy/upf-embb

kubectl logs -n urllc deploy/upf-urllc

kubectl logs -n mmtc deploy/upf-mmtc

# Check PFCP association

tail -5 /var/log/open5gs/smf-embb.log | grep -i pfcp

tail -5 /var/log/open5gs/smf-urllc.log | grep -i pfcp

tail -5 /var/log/open5gs/smf-mmtc.log | grep -i pfcp

# Test with UERANSIM

sudo ./nr-ue -c ue-embb.yaml

ping -I uesimtun0 8.8.8.8

# 9. Rollback Procedure

If issues arise, revert to native UPFs:

# Delete K8s resources

kubectl delete -f /etc/open5gs/k8s/embb/upf-embb.yaml

kubectl delete -f /etc/open5gs/k8s/urllc/upf-urllc.yaml

kubectl delete -f /etc/open5gs/k8s/mmtc/upf-mmtc.yaml

# Re-enable native UPFs

sudo systemctl enable open5gs-upfd-embb open5gs-upfd-urllc open5gs-upfd-mmtc

sudo systemctl start open5gs-upfd-embb open5gs-upfd-urllc open5gs-upfd-mmtc

# 10. Architecture Diagram

Before (native):

UERANSIM UE → gNB → [native UPF process on VM] → ogstun-xxx → internet

↕ PFCP

[native SMF on VM]

After (Kubernetes):

UERANSIM UE → gNB → [UPF Pod in K8s (hostNetwork)] → ogstun-xxx → internet

↕ PFCP (same 127.0.0.x)

[native SMF on VM — unchanged]

Architecture:

┌─────────────────────────────────────────────────┐

│  VM (192.168.49.143)                            │

│                                                 │

│  Native Services (unchanged):                   │

│    AMF, NRF, NSSF, AUSF, UDM, UDR, PCF        │

│    SMF-eMBB  (127.0.0.4:8805)                  │

│    SMF-URLLC (127.0.0.5:8806)                  │

│    SMF-mMTC  (127.0.0.6:8807)                  │

│                                                 │

│  ┌───────────────────────────────────────────┐  │

│  │  Minikube (192.168.58.2, docker driver)   │  │

│  │                                           │  │

│  │  Namespace: embb                          │  │

│  │    └─ upf-embb pod (hostNetwork)          │  │

│  │       PFCP: 127.0.0.7:8805               │  │

│  │       GTP-U: 192.168.49.143:2152          │  │

│  │       tc: htb 100mbit                     │  │

│  │                                           │  │

│  │  Namespace: urllc                         │  │

│  │    └─ upf-urllc pod (hostNetwork)         │  │

│  │       PFCP: 127.0.0.8:8806               │  │

│  │       GTP-U: 192.168.49.144:2152          │  │

│  │       tc: htb 50mbit + netem 1ms          │  │

│  │                                           │  │

│  │  Namespace: mmtc                          │  │

│  │    └─ upf-mmtc pod (hostNetwork)          │  │

│  │       PFCP: 127.0.0.9:8807               │  │

│  │       GTP-U: 192.168.49.146:2152          │  │

│  │       tc: htb 1mbit (ceil 5mbit)          │  │

│  └───────────────────────────────────────────┘  │

└─────────────────────────────────────────────────┘

# Appendix: Files Created/Modified

| File | Action | Purpose |
| --- | --- | --- |
| /etc/open5gs/k8s/Dockerfile | NEW | UPF container image definition |
| /etc/open5gs/k8s/entrypoint.sh | NEW | TUN + NAT + tc setup, then launch UPF |
| /etc/open5gs/k8s/embb/upf-embb.yaml | NEW | K8s ConfigMap + Deployment + Service for eMBB |
| /etc/open5gs/k8s/urllc/upf-urllc.yaml | NEW | K8s ConfigMap + Deployment + Service for URLLC |
| /etc/open5gs/k8s/mmtc/upf-mmtc.yaml | NEW | K8s ConfigMap + Deployment + Service for mMTC |
| /etc/iptables/rules.v4 | MODIFIED | Cleaned ~180 duplicate rules → 15 essential rules |